### Specifications

High efficient thin film panel with CIS technology.

| External dimensions and weight |  |
| --- | --- |
| Length | 1235 mm |
| Width | 641 mm |
| Nominal Weight | 12,4 kg |
| Materials |  |
| PV cell substrate | CIS (Copper, Indium, Selenium) thin-film |
| Cover glass | Low-iron clear tempered glass |
| Encapsulant | EVA (Ethylene Vinyl Acetate) |
| Back face sheet | Weatherproof plastic film (colour black) |
| Frame set | Anodized aluminium alloy (colour black) |
| Junction box | PPE resin (with bypass diode) |
| Output cable | HCV2 mm² (with weatherproof connectors) |
| Electrical Data |  |
| Pm (Nominal peak power) | 65 W - Performance above 90% |
| Ipm (Nominal peak power current) | 1,78 A |
| Vpm (Nominal peak power voltage) | 36,7 V |
| Isc (Nominal short circuit current) | 2,20 A - Performance above 90% |
| Voc (Nominal open circuit voltage) | 52,7 V - Performance +/- 10% |
| Insulation Characteristics |  |
| Insulation resistance | Higher than 500 MOhm, using a 500 V mega-ohm isulation tester |
| Standoff voltage | DC2000V applied during one minute, with a leak current under 0,05 mA, without resulting in insulation breakage or other damage. |
| Standard Operational Condition | as per JIS C 8918 |
| Ambient Temperature | -20C to + 40C |
| Relative humidity | 45% to 95% |